# INDEX

| Experiment No | Experiment Name | Page No |
| --- | --- | --- |
| 1 | Write and execute a machine-language program in MTS86 trainer board to perform the logical AND operation: A AND B=? , A OR B= ?; Where A=02,B=A3. | 2-4 |
| 2 | Write and execute a machine language program in MTS86 trainer board to solve the logical NOT operation, A’ = ? Where A = 02. | 5-6 |
| 3 | Write and execute a machine language program in MTS86 trainer board to perform the logical XOR operation A ⊕ B = ? Where A = 06 and B = 5E. | 7-8 |
| 4 | Write and execute a machine language program in MTS86 trainer board to solve the shift-left operation. E.g. A<<1=? Where A =3C. | 9 |
| 5 | Write and execute a machine language program in MTS86 trainer board to solve the shift-right operation. E.g. A>>1=? Where A=5D. | 10 |
| 6 | Write and execute a program in MTS86 trainer board and show hexadecimal value (0,1, ...F) on the seven-segment display. | 11-12 |
| 7 | Write and execute a program in MTS86 trainer board to display any value on the LED display | 13-14 |
| 8 | Write and execute a program in MTS86 trainer board to interface and display output using the D/A converter. | 15-16 |
| 9 | Write and execute a program in MTS86 trainer board to display output on the LED MATRIX. | 17 |
| 10 |  |  |

Experiment No-01

Experiment Name: Write and execute a machine-language program in MTS86 trainer board to perform the logical AND operation:

A AND B=? , A OR B= ?; Where A=02,B=A3.

Objective: The 8086 kit MTS86 is use to perform machine language program on it to solve any operations. This has I/O facilities and built in with different interfacing devices such as 8255, 8251, 8259, 8253, 8254 etc.

# Specification:

### CPU: 8086

- Display Unit: LCD(16x2)

- RAM: 64kb

- ROM: 64kb

- User Memory: 64kb

Procedure: A AND B

| Assembly Language | Machine Language |
| --- | --- |
| MOV AL, 02 | B0, 02 |
| AND AL, A3 | 24, A3 |

# Truth Table:

| A | B | A AND B |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

INPUT:		A = 02 B = A3

OUTPUT: 	A = 02 	0000 0010

B = A3 	1010 0011

AB = 02 	0000 0010

Procedure: A OR B

| Assembly Language | Machine Language |
| --- | --- |
| MOV AL, 02 | B0, 02 |
| OR AL, A3 | OC, A3 |

# Truth Table:

| A | B | A OR B |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 1 |

| INPUT: | A = 02 |  |
| --- | --- | --- |
|  | B = A3 |  |
| OUTPUT: | A = 02  | 0000 0010 |
|  | B = A3  | 1010 0011 |
|  | AB = 02  | 1010 0011 |

# Experiment No-02

Experiment Name: Write and execute a machine language program in MTS86 trainer board to solve the logical NOT operation, Á = ? Where A = 02.

Objective: The 8086 kit MTS86 is use to perform machine language program on it to solve any operations. This has I/O facilities and built in with different interfacing devices such as 8255, 8251, 8259, 8253, 8254 etc.

# Specification:

### CPU: 8086

- Display Unit: LCD(16x2)

- RAM: 64kb

- ROM: 64kb

- User Memory: 64kb

# Procedure:

| Assembly Language | Machine Language |
| --- | --- |
| MOV AL, 02 | B0, 02 |
| NOT AL | F6, DO |

Truth Table:

| A | Á |
| --- | --- |
| 0 | 1 |
| 1 | 0 |

INPUT:	A = 02

OUTPUT:	A = 02	 0000 0010

Á = FD	 1111 1101

# Experiment No-03

Experiment Name: Write and execute a machine language program in MTS86 trainer board to perform the logical XOR operation A ⊕ B = ? Where A = 06 and B = 5E.

Objective: The 8086 kit MTS86 is use to perform machine language program on it to solve any operations. This has I/O facilities and built in with different interfacing devices such as 8255, 8251, 8259, 8253, 8254 etc.

# Specification:

### CPU: 8086

- Display Unit: LCD(16x2)

- RAM: 64kb

- ROM: 64kb

- User Memory: 64kb

# Procedure:

| Assembly Language | Machine Language |
| --- | --- |
| MOV AL, 06H | B0, 06 |
| MOV BL, 5EH | B3, 5E |
| XOR AL, BL | 30, D8 |

# Truth Table:

| A | B | A ⊕ B |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 0 |

INPUT:	A = 06  0000 0110

B = 5E  0101 1110

OUTPUT:	A = 06 	0000 0110

B = 5E 	0101 1110

AB = 02 	0101 1000

# Experiment No-04

Experiment Name: Write and execute a machine language program in MTS86 trainer board to solve the shift-left operation. E.g. A<<1=? Where A

## =3C

Objective: The 8086 kit MTS86 is use to perform machine language program on it to solve any operations. This has I/O facilities and built in with different interfacing devices such as 8255, 8251, 8259, 8253, 8254 etc.

# Specification:

### CPU: 8086

- Display Unit: LCD(16x2)

- RAM: 64kb

- ROM: 64kb

- User Memory: 64kb

# Procedure:

| Assembly Language | Machine Language |
| --- | --- |
| MOV AL, 3C | B0, 3C |
| SHL AL, 1 | D0, E0 |

| INPUT: | AL = 3C  | 0011 1100 |
| --- | --- | --- |
| OUTPUT: | AL = 3C  | 0011 1100 |
|  | SHL  | 0111 1000 |

# Experiment No-05

Experiment Name: Write and execute a machine language program in MTS86 trainer board to solve the shift-right operation. E.g. A>>1=? Where A=5D.

Objective: The 8086 kit MTS86 is use to perform machine language program on it to solve any operations. This has I/O facilities and built in with different interfacing devices such as 8255, 8251, 8259, 8253, 8254 etc.

# Specification:

### CPU: 8086

- Display Unit: LCD(16x2)

- RAM: 64kb

- ROM: 64kb

- User Memory: 64kb

# Procedure:

| Assembly Language | Machine Language |
| --- | --- |
| MOV AL, 5D | B0, 5D |
| SHR AL, 1 | D0, D8 |

INPUT:	AL = 5D 	0101 1101

OUTPUT: 	AL = 5D 	0101 1101

SHR		0010 1110

# Experiment No-06

## Experiment Name: Write and execute a program in MTS86 trainer board and show hexadecimal value (0,1, ...F) on the seven-segment display.

Objective: The 8086 kit MTS86 is use to perform machine language program on it to solve any operations. This has I/O facilities and built in with different interfacing devices such as 8255, 8251, 8259, 8253, 8254 etc.

# Specification:

### CPU: 8086

- Display Unit: LCD(16x2)

- RAM: 64kb

- ROM: 64kb

- User Memory: 64kb

# Procedure:

| Assembly Language | Machine Language |
| --- | --- |
| MOV AL,1000 0000B | B0, 80 |
| OUT PORT-CR, AL | E6, 1F |
| MOV AL, 1111 0000B | B0, F0 |
| OUT PORT-B, AL | E6, 1B |
| MOV AL, 0000 0000B | B0, 00 |
| OUT PORT-C, AL | E6, 1D |
| MOV AL, C0 | B0, C0 |
| OUT PORT-A, AL | E6, 19 |

Code change for(0,1,2, ……9) as C0/F9/A4/B0/99/82/F8/80/F9…….

# Example:

# Experiment No-07

## Experiment Name: Write and execute a program in MTS86 trainer board to display any value on the LED display.

Objective: The 8086 kit MTS86 is use to perform machine language program on it to solve any operations. This has I/O facilities and built in with different interfacing devices such as 8255, 8251, 8259, 8253, 8254 etc.

# Specification:

### CPU: 8086

- Display Unit: LCD(16x2)

- RAM: 64kb

- ROM: 64kb

- User Memory: 64kb

# Procedure:

| Assembly Language | Machine Language |
| --- | --- |
| MOV AL,1000 0000B | B0, 80 |
| OUT PORT-CR, AL | E6, 1F |
| MOV AL, 1111 1111B | B0, FF |
| OUT PORT-B, AL | E6, 19 |
| MOV AL, 0000 0000B | B0, 00 |
| OUT PORT-C, AL | E6, 1D |
| MOV AL, 1111 0001B | B0, F1 |
| OUT PORT-B, AL | E6, 1B |

Code change ( By SHL): 1111 0001B/1111 0010B/ 1111 0100B/ 1111 1000B

like this. By this way colour will change in different shape.

# OUTPUT:

# Experiment No-08

## Experiment Name: Write and execute a program in MTS86 trainer board to interface and display output using the D/A converter.

Objective: The 8086 kit MTS86 is use to perform machine language program on it to solve any operations. This has I/O facilities and built in with different interfacing devices such as 8255, 8251, 8259, 8253, 8254 etc.

# Specification:

### CPU: 8086

- Display Unit: LCD(16x2)

- RAM: 64kb

- ROM: 64kb

- User Memory: 64kb

# Procedure:

| Assembly Language | Machine Language |
| --- | --- |
| MOV AL,1000 0000B | B0, 80 |
| OUT PORT-CR, AL | E6, 1F |
| MOV AL, 1111 1111B | B0, FF |
| OUT PORT-B, AL | E6, 19 |
| MOV AL, 1111 0000B | B0, F0 |
| OUT PORT-B, AL | E6, 1B |
| MOV AL, 00  05 | B0, 00 |
| OUT PORT-C, AL | E6, 1D |

Code change: 00/ 01/ 05/ 10/ 0F

By this coding the light level will increase or decrease.

# Experiment No-09

## Experiment Name: Write and execute a program in MTS86 trainer board to display output on the LED MATRIX.

Objective: The 8086 kit MTS86 is use to perform machine language program on it to solve any operations. This has I/O facilities and built in with different interfacing devices such as 8255, 8251, 8259, 8253, 8254 etc.

# Specification:

### CPU: 8086

- Display Unit: LCD(16x2)

- RAM: 64kb

- ROM: 64kb

- User Memory: 64kb

# Procedure:

| Assembly Language | Machine Language |
| --- | --- |
| MOV AL,1000 0000B | B0, 80 |
| OUT PORT-CR, AL | E6, 1F |
| MOV AL, 1111 1111B | B0, FF |
| OUT PORT-A, AL | E6, 19 |
| MOV AL, 1111 0000B | B0, F0 |
| OUT PORT-B, AL | E6, 1B |
| MOV AL, 0000 0001B | B0, 01 |
| OUT PORT-C, AL | E6, 1C |